Dossier 1 : Réseaux  informatiques

1)

| de hote | @ Sous reseau | Masque de Reseau |
| --- | --- | --- |
| Site SUD-MARKTING | 192.168.10.0 | 255 .255.255.224 |
| Site SUD- VENTES | 192.168.10.32 | 255 .255.255.224 |
| Site Tanger | 192.168.10.64 | 255 .255.255.224 |
| Site Larache- Production | 192.168.10.96 | 255 .255.255.224 |
| Site Larache- Administration | 192.168.10.128 | 255 .255.255.240 |
| Liason NORD- Tanger | 192.168.10.144 | 255 .255.255.252 |
| LiasonNORD – Larache | 192.168.10.148 | 255 .255.255.252 |
| Liason Tanger – Larache | 192.168.10.152 | 255 .255.255.252 |
| Liason NORD - SUD | 192.168.10.156 | 255 .255.255.252 |

2)

| HOTE | INTERFACE | CONFIGURATION IP (@IP/MASQUE ) |
| --- | --- | --- |
| Routeur SUD | F0/0.20 | 192.168.10.1/27 |
|  | F0/0.30 | 192.168.10.33/27 |
|  | S2/0 | 192.168.10.158/30 |
| Routeur NORD | S2/0 | 192.168.10.145/30 |
|  | S3/0 | 192.168.10.149/30 |
|  | S6/0 | 192.168.10.157/30 |
|  | S7/0 | 212.217.0.1/30 |
| Routeur Larache | F0/0 | 192.168.10.129/28 |
|  | F1/0 | 192.168.10.97/27 |
|  | S2/0 | 192.168.10.150/30 |
|  | S3/0 | 192.168.10.154/30 |
| Routeur Tanger | F0/0 | 192.168.10.65/27 |
|  | S2/0 | 192.168.10.153/30 |
|  | S3/0 | 192.168.10.146/30 |
| PC0 | FastEthernet | 192.168.10.130/28 |
|  | Passerlle | 192.168.10.129 |
| PC1 | FastEthernet | 192.168.10.131/28 |
|  | Passerlle | 192.168.10.129 |
| PC2 | FastEthernet | 192.168.10.98/27 |
|  | Passerlle | 192.168.10.97 |
| PC3 | FastEthernet | 192.168.10.99/27 |
|  | Passerlle | 192.168.10.97 |
| PC4 | FastEthernet | 192.168.10.66/27 |
|  | Passerlle | 192.168.10.65 |
| PC5 | FastEthernet | 192.168.10.67/27 |
|  | Passerlle | 192.168.10.65 |
| PC6 | FastEthernet | 192.168.10.2/27 |
|  | Passerlle | 192.168.10.1 |
| PC7 | FastEthernet | 192.168.10.3/27 |
|  | Passerlle | 192.168.10.1 |
| PC8 | FastEthernet | 192.168.10.34/27 |
|  | Passerlle | 192.168.10.33 |
| PC9 | FastEthernet | 192.168.10.35/27 |
|  | Passerlle | 192.168.10.33 |
| SERVER WEB | FastEthernet | 192.168.10.132 |
|  | Passerlle | 192.168.10.129 |

Partie II :COMMUTATION

4)

SMarketing(config)#interface FastEthernet1/1

SMarketing(config-if)#switchport mode trunk

SVentes(config)#interface FastEthernet1/1

SVentes(config-if)#switchport mode trunk

5)

SMarketing#vlandatabase

SMarketing(vlan)#vtp client

SMarketing(vlan)#vtpdomain var11

Changing VTP domainnamefrom NULL to var11

SVentes#vlandatabase

SVentes(vlan)#vtp server

SVentes(vlan)#vtpdomain var11

Changing VTP domainnamefrom NULL to var11

6)

SVentes(vlan)#vlan 20 name VENTES

VLAN 20 added:

Name: VENTES

SVentes(vlan)#vlan 30 name MARKETING

VLAN 30 added:

Name: MARKETING

7)

SVentes(config)#interface FastEthernet3/1

SVentes(config-if)#switchport mode access

SVentes(config-if)#switchportaccess vlan 20

SVentes(config)#interface FastEthernet2/1

SVentes(config-if)#switchport mode access

SVentes(config-if)#switchportaccess vlan 20

SMarketing(config)#interface FastEthernet2/1

SMarketing(config-if)#switchport mode access

SMarketing(config-if)#switchportaccess vlan 30

SMarketing(config-if)#exit

SMarketing(config)#interface FastEthernet3/1

SMarketing(config-if)#switchport mode access

SMarketing(config-if)#switchportaccess vlan 30

8)

SUD(config)#interface FastEthernet0/0

SUD(config-if)#EX

SUD(config)#interface FastEthernet0/0.20

SUD(config-subif)#ENcapsulation Dot1Q 20

SUD(config-subif)#IP ADdress 192.168.10.1 255.255.255.224

SUD(config-subif)#EX

SUD(config)#interface FastEthernet0/0.30

SUD(config-subif)#ENcapsulation Dot1Q 30

SUD(config-subif)#IP ADdress 192.168.10.33 255.255.255.224

SUD(config-subif)#EX

9)  SVentes(config)#spanning-tree vlan 1 priority 4096

PARTIE III :ROUTAGE

10)NORd(config)#hostname NORD

NORD(config)#ENAblePassword 12345

NORD(config)#LIneVty 04

NORD(config-line)#PASsword 12345

NORD(config-line)#LOGIN

NORD(config-line)#EXIT

NORD(config)#LIneCOnsole 0

NORD(config-line)#PASSword 12345

NORD(config-line)#LOGIN

NORD(config-line)#EXIT

NORD(config)#BANnerMotd # Authorizedaccessonly l #

NORD(config)#loggingsynchronous

11)

NORD(config)#ip route 192.168.10.96 255.255.255.224 192.168.10.150

NORD(config)#ip route 192.168.10.128 255.255.255.240 192.168.10.150

NORD(config)#ip route 192.168.10.64 255.255.255.224 192.168.10.146

NORD(config)#ip route 192.168.10.152 255.255.255.252 192.168.10.146

NORD(config)#ip route 192.168.10.0 255.255.255.224 192.168.10.158

NORD(config)#ip route 192.168.10.32 255.255.255.224 192.168.10.158

Tanger(config)#ip route 192.168.10.0 255.255.255.224 192.168.10.145

Tanger(config)#ip route 192.168.10.32 255.255.255.224 192.168.10.145

Tanger(config)#ip route 192.168.10.148 255.255.255.252 192.168.10.145

Tanger(config)#ip route 192.168.10.156 255.255.255.252 192.168.10.145

Tanger(config)#ip route 212.217.0.0 255.255.255.252 192.168.10.145

Tanger(config)#ip route 192.168.10.96 255.255.255.224 192.168.10.154

Tanger(config)#ip route 192.168.10.128 255.255.255.240 192.168.10.154

SUD(config)#ip route 192.168.10.96 255.255.255.224 192.168.10.157

SUD(config)#ip route 192.168.10.64 255.255.255.224 192.168.10.157

SUD(config)#ip route 192.168.10.128 255.255.255.240 192.168.10.157

SUD(config)#ip route 192.168.10.148 255.255.255.252 192.168.10.157

SUD(config)#ip route 192.168.10.144 255.255.255.252 192.168.10.157

SUD(config)#ip route 192.168.10.152 255.255.255.252 192.168.10.157

SUD(config)#ip route 212.217.0.0 255.255.255.252 192.168.10.157

Larache(config)#ip route 192.168.10.0 255.255.255.224 192.168.10.149

Larache(config)#ip route 192.168.10.32 255.255.255.224 192.168.10.149

Larache(config)#ip route 192.168.10.156 255.255.255.252 192.168.10.149

Larache(config)#ip route 192.168.10.144 255.255.255.252 192.168.10.149

Larache(config)#ip route 212.217.0.0 255.255.255.252 192.168.10.149

Larache(config)#ip route 192.168.10.64 255.255.255.252 192.168.10.153

FAI(config)#ip route 192.168.10.0 255.255.255.224 212.217.0.1

FAI(config)#ip route 192.168.10.32 255.255.255.224 212.217.0.1

FAI(config)#ip route 192.168.10.64 255.255.255.224 212.217.0.1

FAI(config)#ip route 192.168.10.96 255.255.255.224 212.217.0.1

FAI(config)#ip route 192.168.10.128 255.255.255.240 212.217.0.1

FAI(config)#ip route 192.168.10.144 255.255.255.252 212.217.0.1

FAI(config)#ip route 192.168.10.148 255.255.255.252 212.217.0.1

FAI(config)#ip route 192.168.10.152 255.255.255.252 212.217.0.1

FAI(config)#ip route 192.168.10.156 255.255.255.252 212.217.0.1

12)

Tanger(config)#IP ROUte 0.0.0.0 0.0.0.0 192.168.10.145

SUD(config)#IP Route 0.0.0.0 0.0.0.0 192.168.10.157

Larache(config)#IP ROute 0.0.0.0 0.0.0.0 192.168.10.149

NORD(config)#IP Route 0.0.0.0 0.0.0.0 212.217.0.2

13)

NORD#TRaceroute 192.168.10.158

Type escape sequence to abort.

Tracing the route to 192.168.10.158

1   192.168.10.158  20 msec   20 msec   20 msec

SUD#TRaceroute 192.168.10.157

Type escape sequence to abort.

Tracing the route to 192.168.10.157

1   192.168.10.157  20 msec   20 msec   20 msec

14)

NORD(config)#ACCess-list 1 Permit 192.168.1.0 0.0.0.31

NORD(config)#ACCess-list 1 Permit 192.168.1.32 0.0.0.31

NORD(config)#ACCess-list 1 Permit 192.168.1.64 0.0.0.31

NORD(config)#ACCess-list 1 Permit 192.168.1.96 0.0.0.31

NORD(config)#ACCess-list 1 Permit 192.168.1.128 0.0.0.15

NORD(config)#IP NAT INside Source List 1 INterface Serial 7/0 Overload

NORD(config)#interface Serial7/0

NORD(config-if)#IP NAtOUtside

NORD(config-if)#EX

NORD(config)#interface Serial2/0

NORD(config-if)#IP NAt Inside

NORD(config-if)#EX

NORD(config)#interface Serial3/0

NORD(config-if)#IP NAt Inside

NORD(config-if)#EX

NORD(config)#interface Serial6/0

NORD(config-if)#IP NAt Inside

NORD(config-if)#EX

PARTIE IV : ACL

16)

SUD(config)#ACcess-list 101 DenyICmp 192.168.10.0 0.0.0.31 HOST 192.168.10.132

SUD(config)#ACcess-list 101 DenyICmp 192.168.10.32 0.0.0.31 HOST 192.168.10.132

SUD(config)#ACcess-list 101 PErmit IP ANyAny

SUD(config)#INT F 0/0

SUD(config-if)#IP ACcess-group 101 IN

SUD(config-if)#EX

17)

Larache(config)#ACCess-list 1 Deny 192.168.10.96 0.0.0.31

Larache(config)#LINeVty 04

Larache(config-line)#Access-class 1 IN

Larache(config-line)#EXIT

Larache(config)#LIne Console 0

Larache(config-line)#Access-class 1 IN

Larache(config-line)#EXIT

18)

Dossier 2 : Scripting et Administration linux

1 ))))

ifconfig eth0 192.168.1.1 netmask 255.255.255.0

rpm -d bind-9.7.0-9.P1.fc13.i686.rpm

installation de paquetge DNS

rpm -ivh bind-9.7.0-9.P1.fc13.i686.rpm

rpm -ivh bind-chroot-9.7.0-9.P1.fc13.i686.rpm

2)))))

cd /etc

nanonamed.conf

le fichier de configuration principal

_________________________________________________________________

options {

listen-on port 53 { 192.168.1.1; };

listen-on-v6 port 53 { ::1; };

directory	"/var/named";

dump-file	"/var/named/data/cache_dump.db";

statistics-file "/var/named/data/named_stats.txt";

memstatistics-file "/var/named/data/named_mem_stats.txt";

allow-query{ localhost; 192.168.1.1/24; };

recursionyes;

};

logging {

channeldefault_debug {

file "data/named.run";

severitydynamic;

};

};

zone "ABTechnologie.ma" IN {

type master;

file "ABTechnologie.ma.direct";

};

zone "1.168.192.in-addr" IN {

type maser;

file "ABTechnologie.ma.inverse";

};

include "/etc/named.rfc1912.zones";

_________________________________________________________________

3))))

[root@localhost ~]# cd /var/named/

[root@localhostnamed]# nanoABTechnologie.ma.direct

le fichier de configuration de zone direct

GNU nano 2.2.4              Fichier : ABTechnologie.ma.direct

-------------------------------------------------------------

$TTL 1D

@	IN SOA  SERVA.ABTechnologie.ma.  root.ABTechnologie.ma. (

0	; serial

1D	; refresh

1H	; retry

1W	; expire

3H )    ; minimum

IN	NS	SERVA.ABTechnologie.ma.

SERVA   IN	A	192.168.1.1

PC02    IN	A	192.168.1.2

PC03    IN	A	192.168.1.3

--------------------------------------------------------------

4))))

[root@localhostnamed]# cd /var/named/

[root@localhostnamed]# nanoABTechnologie.ma.inverse

le fichier de configuration de zone direct

GNU nano 2.2.4              Fichier : ABTechnologie.ma.inverse

_________________________________________________________________

$TTL 1D

@	IN SOA  SERVA.ABTechnologie.ma.  root.ABTechnologie.ma. (

0	; serial

1D	; refresh

1H	; retry

1W	; expire

3H )    ; minimum

IN	NS        @

1	IN	PTR	  SERVA.ABTechnologie.ma.

2	IN	PTR	  PC02.ABTechnologie.ma.

3	IN	PTR	  PC03.ABTechnologie.ma.

_________________________________________________________________

5)))))

[root@localhostnamed]# servicenamed restart

6))))))

[root@localhostnamed]# nslookup

> www.ABTechnologie.ma

> SERVA.ABTechnologie.ma

> PC02.ABTechnologie.ma